Raspberry Pi 4 Project — Setup Log

Record of all steps completed so far, with credentials and configuration

# Credentials & Device Identity

Note: This document contains plaintext passwords. Keep it private — do not upload it to any shared or public location (e.g. public GitHub repo, shared drive without access control).

| Field | Value |
| --- | --- |
| Device | Raspberry Pi 4 |
| Hostname | MSraspi4 (connect via msraspi4.local, lowercase) |
| Pi Username | msproject |
| Pi Password | cummins123 |
| Wi-Fi SSID | ms |
| Wi-Fi Password | lalala11 |
| Wi-Fi Country | IN |
| OS Flashed | Raspberry Pi OS (64-bit) |
| SSH | Enabled during flashing (password authentication) |

# Steps Completed So Far

PHASE 1: Laptop Software Setup (VS Code)

- VS Code installed on laptop

- Remote - SSH extension installed

- Remote Development extension pack installed

- Remote - SSH: Editing Configuration Files (kept, untouched)

- Remote Explorer (kept, untouched)

- SSH client confirmed working via ssh -V

- Git installed and configured (user.name and user.email set globally)

- Local sanity check completed — Python (print Hello World) and C (gcc compile + run) both tested and working directly on the laptop before touching the Pi

PHASE 2: Flashing the OS onto the microSD Card

- Used a Sandisk Ultra 32GB microSD card with a USB card reader

- Windows prompted to format drive D: on first insert — this was skipped (Cancel), since Raspberry Pi Imager overwrites the card regardless

- Raspberry Pi Imager used to flash Raspberry Pi OS (64-bit) onto the card

- Device selected: Raspberry Pi 4

- Advanced options (⚙) configured before writing:

  - Hostname set to MSraspi4

  - SSH enabled with password authentication — username msproject, password cummins123

  - Wireless LAN configured — SSID "ms", password "lalala11", country IN

  - Network type chosen: Secure network (WPA2), not open network — required since GPIO sensor data will be transmitted and needs to stay private/tamper-proof

  - Hidden SSID left unchecked (network broadcasts normally)

- Card written and verified successfully

PHASE 3: First Boot

- microSD card inserted into the Pi 4

- USB-C power connected — Pi booted automatically, fully headless (no monitor/keyboard used)

- Waited ~1–3 minutes for the Pi to expand its filesystem, join the "ms" Wi-Fi network, and start the SSH service

PHASE 4: Connecting from the Laptop (SSH + VS Code Remote-SSH)

Plain SSH test:

ssh msproject@msraspi4.local

Connected successfully after accepting the host fingerprint and entering the password.

VS Code Remote-SSH connection:

- Ctrl+Shift+P → Remote-SSH: Connect to Host → entered msproject@msraspi4.local

- SSH config file updated at C:\Users\pb_sh\.ssh\config with Host / HostName / User entries

- Remote platform selected: Linux

## Issue Encountered & Resolved

First connection attempt failed with an extension compatibility error:

Could not establish connection to "msraspi4.local": Extension 'ms-vscode-remote.remote-ssh'

      CANNOT use API proposal: terminalRemoteResolver...

Cause: Remote - SSH extension had been auto-updated to a pre-release version (0.126.0) incompatible with the installed stable VS Code build (1.134.0).

Fix applied:

- Uninstalled Remote Development (pack) — reinstalling it alone did not resolve the issue, since it does not force-reinstall already-installed dependency extensions

- Uninstalled Remote - SSH directly

- Closed VS Code completely and reopened it

- Reinstalled Remote - SSH — confirmed back on a stable release version (0.124.0), with "Switch to Pre-Release Version" now shown as the option (confirming stable is active)

- Reinstalled Remote Development

- Retried the connection — succeeded

## Successful Connection Confirmed

- Status bar showed SSH: msraspi4.local

- Password entered (cummins123) and accepted

- Opened folder on the Pi — confirmed remote filesystem access

- Terminal prompt confirmed running on the Pi:

msproject@MSraspi4:~ $

Verified installed toolchain on the Pi:

python3 --version   → Python 3.13.5

      gcc --version       → gcc (Debian 14.2.0-19) 14.2.0

      git --version       → confirmed working

PHASE 5: Extensions Installed on the Pi (via Remote-SSH)

| Extension | Purpose |
| --- | --- |
| Python (Microsoft) | Core Python language support — syntax highlighting, IntelliSense, run/debug |
| Pylance | Faster, richer autocomplete and type-checking alongside the Python extension |
| Python Debugger | Breakpoints and step-through debugging for Python running on the Pi |
| C/C++ (ms-vscode.cpptools) | Syntax highlighting, IntelliSense, and debugging for C code |

Note: Installed via “Install in SSH: msraspi4.local” so they run on the Pi itself, not the laptop. Skipped Code Runner (redundant) and third-party Python extensions (avoid conflicting with the official Microsoft ones).

PHASE 6: Project Dependencies Installed on the Pi

Base system update:

sudo apt update && sudo apt upgrade -y

Pip (Python package manager):

sudo apt install python3-pip -y

## Issue Encountered & Resolved: externally-managed-environment

Running pip3 install directly failed with:

error: externally-managed-environment

      This environment is externally managed

Cause: newer Raspberry Pi OS (Debian 12 "Bookworm" and later) blocks pip from installing directly into the system Python, since apt owns that environment and an unmanaged pip install could silently break OS-level tools that depend on it (PEP 668).

Fix applied — used a Python virtual environment (venv), which is a separate, isolated Python install just for this project, so pip installs never touch the system Python:

Create the virtual environment (one-time only):

python3 -m venv ~/gpio-env

Activate the virtual environment (every new session, before running project scripts):

source ~/gpio-env/bin/activate

Prompt changes to confirm it's active:

(gpio-env) msproject@MSraspi4:~ $

Install all project dependencies (inside the venv):

pip3 install gpiozero RPi.GPIO flask paho-mqtt

Exit the virtual environment when done working:

deactivate

Note: Also relevant for sensors using I2C communication (not yet installed, install if/when needed): sudo apt install python3-smbus i2c-tools -y

## Installed Packages — Confirmed (pip3 list, inside gpio-env)

| Package | Version |
| --- | --- |
| blinker | 1.9.0 |
| click | 8.5.0 |
| colorzero | 2.0 |
| Flask | 3.1.3 |
| gpiozero | 2.0.1.post3 |
| itsdangerous | 2.2.0 |
| Jinja2 | 3.1.6 |
| MarkupSafe | 3.0.3 |
| paho-mqtt | 2.1.0 |
| pip | 25.1.1 |
| RPi.GPIO | 0.7.1 |
| setuptools | 84.0.0 |
| Werkzeug | 3.1.8 |

| Package | Purpose |
| --- | --- |
| gpiozero | Simpler, higher-level GPIO pin interface — recommended for most sensor/pin work |
| RPi.GPIO | Lower-level, more direct pin control for precise timing |
| flask | Lightweight web server — lets the Pi serve GPIO data over HTTP to tablet/laptop |
| paho-mqtt | MQTT client library — lightweight protocol suited to logging sensor data to a central gateway |

# VS Code Interpreter Note

After creating the venv, VS Code may prompt to select it as the Python interpreter for the project — accept this so IntelliSense and the Run button use gpio-env automatically. If it doesn't prompt: Ctrl+Shift+P → Python: Select Interpreter → choose the path ~/gpio-env/bin/python3.

# Network Notes

- Chosen network type: Secure (WPA2/WPA3), not open — required for data privacy and integrity since GPIO/sensor data is transmitted over this network

- Pi, laptop/tablet, and the central gateway must all be on the same Wi-Fi network (SSID "ms") to communicate directly

- The Pi only auto-connects to the SSID configured during flashing — changing networks later requires reconfiguration via raspi-config or nmtui over SSH (or Ethernet as a fallback if Wi-Fi is unreachable)